# 0.

| - Who is the primary point of contact for your lab? |  |
| --- | --- |
| - Does your lab personnel occupy/work in any other rooms? |  |
| - Select the hazards present in the lab. |  |
| ☐Acute toxins ☐Carcinogens (1, 2A, 2B) ☐Reproductive toxins (1A, 1B, 2) ☐Oxidizers ☐Pyrogens ☐Select toxins ☐Corrosives ☐Biological Materials (including infectious agents, recombinant materials, and toxins) ☐Creation of Transgenic Animals ☐Class III Lasers ☐Class IV Lasers ☐Radiation ☐Noise ☐Cryogens ☐Flammable gases ☐Others ☐N/A |  |
| - Do you work with live animals in this lab space? (If so, verify proper transportation and containment of animals, and disposal of carcasses. | Yes, No, N/A |
| - Is anesthesia used in this lab? | Yes, No, N/A |
| - What type(s)? ☐Isoflurane ☐Sevoflurane ☐Enflurane |  |
| - Discussed the requirement to report injuries, odors, and other hazardous conditions. | Yes, No, NA |

# 1. Registrations

| - All biological materials present are registered.~ | Yes, No, N/A |
| --- | --- |
| - Personnel lists are accurate, and staff have e-signed registration documents. | Yes, No, N/A |
| - Personnel are current on registration documents.~ | Yes, No, N/A |
| - Personnel have e-signed the registration documents. | Yes, No, N/A |
| - Locations are accurate on registration documents.~ | Yes, No, N/A |
| - All annual reviews of registrations are complete. | Yes, No, N/A |

# 2. Safety Training

| - All lab staff have reviewed the Chemical Hygiene Plan. | Yes, No, N/A |
| --- | --- |
| - Lab Staff are up to date with all applicable specialized training (e.g., shipper, laser safety, other). | Yes, No, N/A |
| - Lab staff are current on shipper training | Yes, No, N/A |
| - Lab staff are current on laser safety training | Yes, No, N/A |
| - Lab staff are current on other required specialized trainings. | Yes, No, N/A |
| - Lab staff are up to date with Lab Safety Training | Yes, No, N/A |
| - Lab staff are up to date with Blood Borne Pathogen Training (if applicable). | Yes, No, N/A |
| - Lab Biosafety Manual is available to staff and annual reviews are documented | Yes, No, N/A |
| - Lab staff have access to the Biosafety Manual | Yes, No, N/A |
| - Annual reviews of the Biosafety Manual are documented | Yes, No, N/A |
| - All lab staff have reviewed the Exposure Control Plan. | Yes, No, N/A |

# 3. Signage & Safety Equipment

| - All applicable signage is displayed in the lab and staff are familiar with their contents (e.g.,1-2-3, Chemical Safety Guide, Waste Disposal Guide). | Yes, No, N/A |
| --- | --- |
| - Emergency 1-2-3 sign is present. | Yes, No, N/A |
| - Staff are familiar with the 1-2-3 procedure. | Yes, No, N/A |
| - Chemical Safety Guide is present in the lab. | Yes, No, N/A |
| - Staff are familiar with the Chemical Safety Guide. | Yes, No, N/A |
| - Waste Disposal Guide is displayed in the lab. | Yes, No, N/A |
| - Staff know how to properly dispose of lab waste. | Yes, No, N/A |
| - Accurate hazard warning signage (i.e., UV, Laser, Radioactive, LAAPP, special equipment (cryogenic, etc.)) has been displayed on lab door, if applicable. | Yes, No, N/A |
| - A UV light warning is displayed on the door. | Yes, No, N/A |
| - Accurate laser signage is displayed on the door. | Yes, No, N/A |
| - A radioactive sign is displayed on the door. | Yes, No, N/A |
| - A Laboratory Animal Allergy Protection Program (LAAPP) sign is displayed on the door. | Yes, No, N/A |
| - Special equipment signage is displayed on the door. | Yes, No, N/A |
| - Handwashing sink, soap, and paper towels are available, and sink is unobstructed. | Yes, No, N/A |
| - Handwashing sink is available. | Yes, No, N/A |
| - Handwashing sink is unobstructed. | Yes, No, N/A |
| - Soap and paper towels are available. | Yes, No, N/A |
| - Eyewash is available, unobstructed, in good working order and documented as flushed weekly. | Yes, No, N/A |
| - An eyewash is available. | Yes, No, N/A |
| - The eyewash is unobstructed. | Yes, No, N/A |
| - The eyewash is documented as flushed weekly (flush on the spot if not). | Yes, No, N/A |
| - Safety shower is available, unobstructed, and documented as flushed within the past 12 months. | Yes, No, N/A |
| - A safety shower is available. | Yes, No, N/A |
| - The safety shower is unobstructed. | Yes, No, N/A |
| - The safety shower is documented as being flushed in the past 12 months. | Yes, No, N/A |
| - Lab equipment is properly labeled to indicate hazards. | Yes, No, N/A |
| - Lab microwave oven(s), refrigerators, freezers clearly labeled "No Food or Drink.” | Yes, No, N/A |
| - Microwave oven(s) clearly labeled “No Food or Drink.” | Yes, No, N/A |
| - Refrigerators clearly labeled “No Food or Drink.” | Yes, No, N/A |
| - Freezers clearly labeled “No Food or Drink.” | Yes, No, N/A |
| - An accurate BSL sign is displayed on the door. | Yes, No, N/A |

# 4. Biological Safety Cabinet (BSC)

| - Biological Safety Cabinet (BSC) has been certified within the last year. | Yes, No, N/A |
| --- | --- |
| BSC date of last certification: ____________ |  |
| - BSC airflow is unobstructed. | Yes, No, N/A |
| - Front grill is unobstructed. | Yes, No, N/A |
| - Rear vents are unobstructed. | Yes, No, N/A |
| - Exhaust filters are unobstructed. | Yes, No, N/A |
| - Top of the BSC is not used for storage. | Yes, No, N/A |
| - BSC interior is free of unnecessary stored items. | Yes, No, N/A |
| - There is no evidence of the use of open flames/flammable gases inside of the BSC. | Yes, No, N/A |
| - Open flames are not used inside the BSC. | Yes, No, N/A |
| - Flammable gases are not used inside the BSC. | Yes, No, N/A |
| - Sash is at the designated heights or lower | Yes, No, N/A |
| - Potentially infectious agents are manipulated within a BSC. | Yes, No, N/A |

# 5. Chemical Fume Hood (CFH) & Other Local Exhaust

| - Chemical Fume Hood has been certified within the last year. | Yes, No, N/A |
| --- | --- |
| Date of last certification: _____________ |  |
| - Air foil and rear baffle are unobstructed. | Yes, No, N/A |
| - Air foil is unobstructed. | Yes, No, N/A |
| - Rear baffle is unobstructed. | Yes, No, N/A |
| - CFH is free of unnecessary stored items. | Yes, No, N/A |
| - CFH airflow is unobstructed. | Yes, No, N/A |
| - Sash is at the designated height or lower. | Yes, No, N/A |
| - All other required local exhaust certification has been obtained (i.e., downdraft table, histology cabinet, elephant trunks, animal racks, autoclave vents, and slot hoods). | Yes, No, N/A |

# 6. Gas Cylinders

| - All cylinders are properly secured and labeled; caps are on all reserve cylinders, and there are no excess cylinders stored in the lab. | Yes, No, N/A |
| --- | --- |
| - All gas cylinders are properly secured. | Yes, No, N/A |
| - All gas cylinders are properly labeled. | Yes, No, N/A |
| - Reserve cylinders are capped. | Yes, No, N/A |
| - There are no excess cylinders stored in the lab. | Yes, No, N/A |

# 7. Fire Protection

| - Sprinkler heads are free of obstructions with an 18" clearance. | Yes, No, N/A |
| --- | --- |
| - Lab is free of open penetrations (walls, ceilings, floor, etc.) | Yes, No, N/A |
| - Doors to labs are kept closed to ensure fire protection and negative directional air flow from general occupancy, corridor, and office areas. | Yes, No, N/A |
| - Doors to hazardous storage (e.g., flammable, corrosive cabinets) are kept closed to ensure protection from fire. | Yes, No, N/A |
| - All exits, passage routes, and corridors within the lab are unobstructed (i.e., have the appropriate clearance for egress based on building requirements). | Yes, No, N/A |
| - Exits are unobstructed. | Yes, No, N/A |
| - Passage routes are unobstructed. | Yes, No, N/A |
| - Corridors are unobstructed. | Yes, No, N/A |

# 8. Electrical

| - Equipment is free of frayed/damaged wiring. | Yes, No, N/A |
| --- | --- |
| - Outlets, electrical strips, and extension cords used in the lab aren’t overloaded or "daisy-chained." | Yes, No, N/A |
| - Outlets used in the lab are not overloaded. | Yes, No, N/A |
| - Electrical strips used in the lab are not overloaded. | Yes, No, N/A |
| - Extension cords used in the lab are not overloaded. | Yes, No, N/A |
| - Electrical strips/extension cords are not “daisy-chained.” | Yes, No, N/A |
| - There are no permanent extension cords. | Yes, No, N/A |
| - All electrical equipment (e.g., computers/power strips) is raised off the floor. | Yes, No, N/A |
| - Electrical outlets within 1 meter of water sources are protected by G.F.C.I. (Ground Fault Circuit Interrupter) | Yes, No, N/A |
| - There are no portable space heaters. | Yes, No, N/A |
| - Electrical panels and breakers are unobstructed (36-inch clearance). | Yes, No, N/A |
| - Electrical panels are unobstructed. | Yes, No, N/A |
| - Electrical breakers are unobstructed. | Yes, No, N/A |

# 9. Chemicals

| - Lab has written SOPs for any Particularly Hazardous Substances (PHS’s)/ hazardous chemicals (acute toxicity, select carcinogens, select reproductive toxins (1A, 1B, 2), corrosives, oxidizers, pyrogens, etc.) used by staff. | Yes, No, N/A |
| --- | --- |
| - Peroxide formers are dated upon receipt and opening; they are stored, tested and/or discarded in accordance with the NIH Chemical Hygiene Plan. | Yes, No, N/A |
| - Peroxide formers are dated upon receipt. | Yes, No, N/A |
| - Peroxide formers are dated upon opening. | Yes, No, N/A |
| - Peroxide formers are stored in accordance with the NIH CHP. | Yes, No, N/A |
| - Peroxide formers are tested in accordance with the NIH CHP. | Yes, No, N/A |
| - Peroxide formers are discarded in accordance with the NIH CHP. | Yes, No, N/A |
| - All flammables are stored in a flammable storage cabinet. (Max 500mL) allowed for working stock out of flammable storage cabinet) | Yes, No, N/A |
| - Refrigerated flammables are stored only in hazard rated units. | Yes, No, N/A |
| - Chemicals have been properly segregated by hazard class and in accordance with the NIH Chemical Hygiene Plan. | Yes, No, N/A |
| - Chemicals have been segregated by hazard class. | Yes, No, N/A |
| - Chemicals have been segregated in accordance with the NIH CHP. | Yes, No, N/A |
| - No hazardous materials are stored above eye level. | Yes, No, N/A |
| - All chemical containers within lab are securely closed. | Yes, No, N/A |
| - All chemical containers are properly labeled with name of chemical, date, and associated hazards. | Yes, No, N/A |
| - Corrosives are stored in corrosion resistant and ventilated cabinet(s), in secondary containment, and never under sinks or with aqueous solutions. | Yes, No, N/A |
| - Corrosives are stored in proper cabinet (e.g., corrosion resistant, ventilated). | Yes, No, N/A |
| - Corrosives are stored in secondary containers. | Yes, No, N/A |
| - Corrosives are not stored under the sink. | Yes, No, N/A |
| - Corrosives are not stored with incompatible chemicals (e.g., aqueous solutions). | Yes, No, N/A |
| - There is evidence that staff are familiar with special handling requirements for ASP section K chemicals (if applicable). | Yes, No, N/A |

# 10. Waste Management

| - DEP and DRS are contacted to obtain waste management information before generating multi-hazardous waste. | Yes, No, N/A |
| --- | --- |
| - Lab waste is collected, stored, and disposed of in accordance with the NIH Waste Disposal Guide, Chemical Safety Guide and, if applicable, off-site waste disposal policies for satellite locations. | Yes, No, N/A |
| - Lab waste is collected in accordance with NIH Waste Disposal Guide, Chemical Safety Guide and, if applicable, off-site waste disposal policies for satellite locations. | Yes, No, N/A |
| - Lab waste is stored in accordance with NIH Waste Disposal Guide, Chemical Safety Guide and, if applicable, off-site waste disposal policies for satellite locations. | Yes, No, N/A |
| - Lab waste disposed in accordance with NIH Waste Disposal Guide, Chemical Safety Guide and, if applicable, off-site waste disposal policies for satellite locations. | Yes, No, N/A |
| - All instruments or analyzers that are connected directly to the drain have been approved by DEP (or equivalent). | Yes, No, N/A |
| - Sharps containers are used for sharps only and disposed of when ¾ full as MPW or through Chemical Waste Services/DEP. | Yes, No, N/A |
| - Sharps containers used for sharps only. | Yes, No, N/A |
| - Sharps containers in use are less than ¾ full. | Yes, No, N/A |
| - Sharps containers are disposed of as MPW or through Chemical Waste Services. | Yes, No, N/A |
| - There is no presence of uncapped or otherwise exposed sharps. | Yes, No, N/A |
| - Chemical waste is labeled (including start date). | Yes, No, N/A |
| - Chemical waste containers present have not been in use for more than 60 days | Yes, No, N/A |
| - Waste containers on the benchtop have secondary containment. | Yes, No, N/A |
| - Waste containers on the floor have secondary containment large enough to contain the liquid in the waste container if it leaked. | Yes, No, N/A |
| - Toner cartridges and pipette tip racks are separated and recycled in accordance with the NIH Waste Disposal guide. | Yes, No, N/A |
| - Lab staff is aware of the NIH Environmental Management System Website (NEMS). | Yes, No, N/A |
| - Biohazardous waste is chemically decontaminated or autoclaved and disposed as MPW, if indicated in the IBC-approved registration document.~ | Yes, No, N/A |
| - MPW boxes/containers are not filled past ¾ (40 lbs. max.). | Yes, No, N/A |
| - MPW boxes/containers are double bagged with supplied bags. | Yes, No, N/A |

# 11. General Practices & Procedures

| - There is evidence of the use of safer sharps devices (plastic razor blades, plastic pipettes, blade holders, blunt needles, etc.) | Yes, No, N/A |
| --- | --- |
| - Appropriate disinfectant is used for organism(s) and work surfaces. | Yes, No, N/A |
| - Absorbent pads are clean. | Yes, No, N/A |
| - Vacuum lines have in-line filters (HEPA or equivalent efficiency). | Yes, No, N/A |
| - There is no evidence of consumption of food and drinks in lab areas. | Yes, No, N/A |
| - Furniture items within the lab are non-porous and cleanable or have a non-porous covering (i.e., no cloth chairs, rugs, cloth bulletin boards). | Yes, No, N/A |
| - No slip, trip, or fall hazards in the lab (e.g., glass, water, cords on floor). | Yes, No, N/A |
| - Non-research related plants are not present in the lab. | Yes, No, N/A |
| - Non-research related animals are not present in the lab. | Yes, No, N/A |
| - No discrepancies identified in the biological inventory during spot checks. | Yes, No, N/A |
| - Unbreakable secondary containment with hazard labels is present for transport of biological materials outside of the lab. | Yes, No, N/A |

# 12. Personal Protective Equipment

| - Occupants are properly attired (e.g., no exposed skin, closed toed shoes). | Yes, No, N/A |
| --- | --- |
| - Lab staff have been trained on proper procedures for wearing PPE outside of the lab (i.e., one glove rule, proper route) | Yes, No, N/A |
| - Personal protective equipment is available (e.g., gloves, lab coats, goggles/face shield, For registered labs only: disposable solid front, liquid resistant gowns/aprons, etc.). | Yes, No, N/A |
| - All personal protective equipment required for work being performed in the lab is worn by staff (e.g., gloves, lab coats, goggles/face shield, For registered labs only: disposable solid front, liquid resistant gowns/aprons, etc.). | Yes, No, N/A |
| - Gloves or double gloves (if dictated by registration documents) are worn by staff. | Yes, No, N/A |
| - Lab coats/solid front gowns (if dictated by registration documents) are worn by staff. | Yes, No, N/A |
| - Eye/face protection is worn by staff when required. | Yes, No, N/A |
| - Personnel required to use respiratory protection are enrolled in the Respiratory Protection Program. | Yes, No, N/A |

# 13. Cold Room

| - The cold room door has an emergency release. | Yes, No, N/A |
| --- | --- |
| - The cold room door has a proper seal. | Yes, No, N/A |
| - The cold room is free of paper products. | Yes, No, N/A |
| - The cold room is free of mold. | Yes, No, N/A |
| - A current Cold Room Information Sheet (POC sign) is posted on the door. | Yes, No, N/A |
| - The Preventing Mold Growth sign is posted on the door. | Yes, No, N/A |

# 14. Bite, Scratch, Splash, Exposure (BSSE) Kits – Old World NHPs

| - BSSE SOP for the handling of NHP unfixed tissues, blood, and/or body fluid related exposures is approved (by the IC Safety Committee and DOHS).~ | Yes, No, N/A |
| --- | --- |
| - Staff are aware of the BSSE kit (location and utility).~ | Yes, No, N/A |
| - Staff know how to get or who to contact for replacement valacyclovir upon expiration or use.~ | Yes, No, N/A |
| - The BSSE kit is accessible to all personnel working where NHP tissues, blood, and body fluids are used. | Yes, No, N/A |
| - Valacyclovir is stored between 55°-77°F. | Yes, No, N/A |
| - Valacyclovir is within expiration date. | Yes, No, N/A |
| - Signage is present in the room indicating the presence of the BSSE kit. | Yes, No, N/A |
| - BSSE kit components are not expired. | Yes, No, N/A |

# 15. Centrifuges

| - Containment devices (safety cups or sealed rotor) have properly fitted O-rings in place. | Yes, No, N/A |
| --- | --- |
| - Safety cups and rotors used with biohazardous materials are only opened within a BSC. | Yes, No, N/A |

# 16. Lasers

| - NOTICE laser-controlled area sign(s) posted during periods of service on fully enclosed equipment containing Class 3B or Class 4 services. | Yes, No, N/A |
| --- | --- |
| - DANGER or WARNING laser-controlled area sign posted in areas containing Class 3B or 4 lasers. | Yes, No, N/A |
| - Appropriate laser safety eyewear is present. | Yes, No, N/A |
| - Appropriate laser safety eyewear is being used by staff. | Yes, No, N/A |

# 17. Other

| - Cryogens are stored in approved posted areas. | Yes, No, N/A |
| --- | --- |
| - Cryogen PPE is available (e.g., face shield, cryogenic loose-fitting gloves, goggles, etc.). | Yes, No, N/A |
| - Cryogen PPE is being used by staff. | Yes, No, N/A |
| - Asphyxiants are only used and stored in ventilated rooms. | Yes, No, N/A |
| - Asphyxiants are only used in ventilated rooms. | Yes, No, N/A |
| - Asphyxiants are only stored in ventilated rooms. | Yes, No, N/A |
| - Hazardous gases are only used and stored in ventilated rooms. | Yes, No, N/A |
| - Hazardous gases are only used in ventilated rooms. | Yes, No, N/A |
| - Hazardous gases are only stored in ventilated rooms. | Yes, No, N/A |
| - PI (or lab representative) made themselves available to the safety representative during or after the survey. | Yes, No, N/A |
| - Survey reflects all observed safety findings. (If "no", state observed safety findings in text box). | Yes, No, N/A |